# SEU 数据集文献全库扫描

——最合适的对比条目

摘要

本报告对公开文献中使用了东南大学（SEU）DDS 驱动系统动态模拟器数据集的实验做了一次全库扫描，覆盖 MDPI、IEEE Xplore、Wiley、Springer、IOP、Nature、ACM 与中文核心期刊，共筛出 27 篇可直接引用的文献。筛选口径为导师提出的"同数据库 · 同任务 · 同数据量 · 同划分 · 同验证"，其中"同任务"定义为单工况 5 分类。按"同库 + 同任务 + 划分明确 + 有基线数值"四项过滤后，得出 8 篇最值得使用的文献：齿轮箱侧以 DDGF-Net（Machines 2026，唯一做了 5 折交叉验证）、DEFT-CNN（IEEE TIM 2021，KNN/SVM 仅 88%）、AMMMP 2024 为首选；轴承侧以 Dual-Channel 多模态融合（Machines 2025）为首选。报告同时标出了必须加条件才能引用的低分条目、仅可作量级参照的条目，以及必须撤下或慎用的条目。

关键词：东南大学数据集；齿轮箱故障诊断；轴承故障诊断；文献对比；可比性

#### 目 录

- 组织口径与结论先行

- 齿轮箱 · 单工况 5 分类（主战场）

- 轴承 · 单工况 5 分类

- 低分但必须加条件的条目

- 同库 · 任务不同（仅作量级参照）

- 必须撤下 / 慎用

- 出处登记

## 一、组织口径与结论先行

本轮检索的目标只有一个：把全网真正用了东南大学（SEU）DDS 数据集的实验找齐，并按"与本文任务的可比性"排序。筛选沿用导师的五条口径——同数据库 · 同任务 · 同数据量 · 同划分 · 同验证，其中"同任务"本文定义为单工况 5 分类（与本文的齿轮 / 轴承任务一致）。

对照模型的性质分两类，表中已标注：【自跑】指对照模型由原文作者在同一套数据与设置下自行训练评测，即导师要的 Type A；【转引】指直接引用他人论文数值。前者可直接进主表，后者必须加"设置差异"列。

按"同库 + 同任务 + 划分明确 + 有基线数值"四项筛完，最值得用的 8 篇如下。

| 优先级 | 文献 | 为什么最合适 | 所在章节 |
| --- | --- | --- | --- |
| ★★★ | DDGF-Net （ Machines 2026 ） | 唯一做了 5 折交叉验证 的同任务文献； 8 个基线全部自跑 | 2.1 |
| ★★★ | DEFT-CNN （ IEEE TIM 2021 ） | 仪器仪表顶刊；两工况 分列报数 ； KNN / SVM 仅 88% | 2.2 |
| ★★★ | AMMMP 2024 | 20-0 单工况、 800 / 200 、结构最贴合本文 | 2.3 |
| ★★ | CBAM-ResNeXt50 （ Sensors 2023 ） | 5 类两工况； AlexNet 仅 93% | 2.4 |
| ★★ | CNN-DOA-LSSVM （ Information 2026 ） | 20-0 、 5 类、 7:3 ，划分交代清楚 | 2.5 |
| ★★ | CS-LAMRNet （ Mathematics 2025 ） | VGG11 90.58% ，且 报了方差 | 2.6 |
| ★★ | ConvFormer-SENet （ IEEE 2025 ） | 20-0 、 5 类、 1023 样本 / 类、 7:3 | 2.7 |
| ★★ | Dual-Channel 多模态融合（ Machines 2025 ） | 轴承侧 5 类单工况，自跑基线 + 报标准差 | 3.1 |

## 二、齿轮箱 · 单工况 5 分类（主战场）

### 2.1  DDGF-Net ★★★ —— 唯一有交叉验证的

Ma R., Wang X., Cheng J., Xie T., Li S., Wang C. DDGF-Net: A Novel Dual-Domain Generative-Discriminative Fusion Network for Gearbox Fault Diagnosis Under Strong Noise Conditions. Machines, 2026, 14(9): 1013. DOI: 10.3390/machines14091013

任务：SEU 齿轮箱 5 类（正常 / 齿面崩裂 / 断齿 / 齿根裂纹 / 齿面磨损），恒定工况 20 Hz-0 V，采样 5120 Hz。

设置：样本长 1024 点、无重叠滑窗；8:2 分层划分（seed 42）+ 5 折分层交叉验证；加性高斯白噪声，训练与测试保持同一 SNR。

| 方法 | −10 dB | −8 dB | −6 dB | −4 dB | 0 dB | 6 dB | 10 dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MLP （ 8 个手工特征） | 21–26 | — | — | — | — | — | — |
| ViT-1D | 36.1 | — | — | — | — | 84.9 | 88.0 |
| BiLSTM-Attention | 36.9 | — | — | — | — | — | — |
| ResNet-1D | 56.3 | — | — | 92.4 | — | — | — |
| DRSN | 56.5 | — | 83.1 | 92.5 | — | — | — |
| WDCNN | — | — | — | — | — | — | — |
| STFT-2D | — | — | — | — | — | — | — |
| DConformer | — | — | — | — | — | — | — |
| DDGF-Net（本文方法） | 71.2 | 84.3 | 93.5 | — | ≈100 | — | — |

原文明确写道："All baseline models were evaluated under identical conditions: the same data split (seed 42, 8:2 train/test, five-fold stratified cross-validation), the same SNR levels, and the same accuracy metric."——这是 A 类自跑基线，且带 5 折交叉验证，正对导师要求。注意原文 Table 4 未公开完整数值矩阵，上表中 WDCNN / STFT-2D / DConformer 在部分 SNR 下无明确数字，引用时只写正文明确给出的那些。

### 2.2  DEFT-CNN ★★★ —— 顶刊，两工况分列

Zhu C., Chen Z., Zhao R., Wang J., Yan R. Decoupled Feature-Temporal CNN: Explaining Deep Learning-Based Machine Health Monitoring. IEEE Transactions on Instrumentation and Measurement, 2021, 70: 1–13. DOI: 10.1109/TIM.2021.3084310

| 方法 | 20 Hz-0 V | 30 Hz-2 V |
| --- | --- | --- |
| KNN | 88.37 | 88.24 |
| SVM | 88.49 | 87.61 |
| LFGRU | 94.23 | 95.89 |
| Sparse Filtering | 98.56 | 98.29 |
| DEFT-CNN | 98.40 | 98.12 |
| DEFT-CNN + 空间注意力 | 99.61 | 99.41 |

设置：采样 1024 Hz；512 s 窗、原始长 5120 切 20 个 256 点子窗，每样本形成 20×9 特征矩阵；随机 80/20 划分；X / Y / Z 三向振动。无交叉验证。这是目前档次最高、最干净的一条——IEEE TIM 顶刊，KNN / SVM 只有 88%，与本文差距 9~11 个点。

### 2.3  AMMMP 2024 ★★★ —— 结构最贴合

Fault Diagnosis of Gearbox Based on CNN and LSTM with Attention Mechanism. Highlights in Science, Engineering and Technology, 2024, 93: 291–292.

| 方法 | 准确率 |
| --- | --- |
| LSTM | 90.75 |
| CNN | 92.90 |
| CNN-LSTM | 94.82 |
| CNN-AM （本文方法） | 96.28 |
| 本文（SEU 齿轮箱） | 97.69~99.85 |

设置：只取 20 Hz-0 V 一个工况、5 类（NC / Chipped / Miss / Root / Surface），每类 800 训练 / 200 测试，随机 8:2，30 epoch，Adam，10 次重复。无交叉验证。

两个必须写明的条件：① 该文自己报的最好结果为 98.5%，高于本文齿轮下限 97.69%，因此只能写"优于其列出的基线"，不能写"优于该文献"；② 出处为 Darcy & Roy Press 会议论文集，层级偏低，不宜作主要论据。

### 2.4  CBAM-ResNeXt50 ★★

Wang Z., Tang Y., Dong Y., et al. Optimization of Gearbox Fault Detection Method Based on Deep Residual Neural Network Algorithm. Sensors, 2023, 23(17): 7573. DOI: 10.3390/s23177573

| 方法 | 精度 · 工况 1 | 精度 · 工况 2 | 召回 · 工况 1 | 召回 · 工况 2 |
| --- | --- | --- | --- | --- |
| CBAM-ResNeXt50 （本文） | 1.0000 | 1.0000 | 1.0000 | 1.0000 |
| DenseNet121 | 0.9741 | 0.9778 | 0.9738 | 0.9776 |
| ResNeXt50 | 0.9747 | 0.9864 | 0.9738 | 0.9863 |
| ResNet50 | 0.9699 | 0.9655 | 0.9687 | 0.9650 |
| AlexNet | 0.9322 | 0.9152 | 0.9325 | 0.9113 |

原文报的是 precision / recall，不是 accuracy，引用时须写对指标名。本文方法测试准确率为 100% / 99.875%，平均训练时间 22.6 / 23.1 s。

### 2.5  CNN-DOA-LSSVM ★★

Fault Classification Method for Rotating Machinery Based on Hybrid Model of CWT and CNN-DOA-LSSVM. Information, 2026, 17(6): 580. DOI: 10.3390/info17060580

| 方法 | 准确率 |
| --- | --- |
| CNN | 98.22 |
| CNN-DOA-LSSVM （本文方法） | 99.78 |

设置：20 Hz-0 V、行星齿轮箱 Z 向、5 类；丢弃前 1000 点，窗长 2048、步长 1000；每类 300 样本（共 1500）；7:3 划分；CWT 转 64×64 输入 CNN。差距仅 1.5 个点，必须写清设置。

### 2.6  CS-LAMRNet ★★ —— 报方差

Gearbox Fault Diagnosis Based on Compressed Sensing and Multi-Scale Residual Network with Lightweight Attention Mechanism. Mathematics, 2025, 13(9): 1393. DOI: 10.3390/math13091393

| 方法 | 平均准确率 | 方差 |
| --- | --- | --- |
| VGG11 | 90.58% | 0.8563 |
| MobileNet V3 | 96.50% | 0.1464 |
| ResNet18 | 96.60% | 0.1326 |
| GADF-CNN | 97.70% | 0.0976 |
| MTF-ResNet | 97.80% | 0.0876 |
| ConvNeXt-T | 99.10% | 0.0064 |
| DRSN-CW | 99.60% | 0.0045 |
| CS-LAMRNet （本文方法） | 100% | 0 |

设置：1000 点滑窗，GADF 转 224×224 图，每类 800 训练 / 200 测试，8:2 随机划分。无交叉验证，但报了方差（说明做了多次重复），规范性较好。

### 2.7  ConvFormer-SENet ★★

Hierarchical Convolution-Transformer Framework for Gear Fault Diagnosis Under Severe Noise. IEEE Xplore, 2025, 文献号 11037744.

| 方法 | 准确率 |
| --- | --- |
| ResNet50 | 96.8 |
| CNN-CBAM | 97.0 |
| UniFormer | 98.0 |
| ConvFormer-SENet （本文方法） | 100 |

设置：行星齿轮箱 x + y、1200 r/min、0 V；窗长 1024 滑窗，每类 1023 样本；7:3 划分；150 epoch。

### 2.8  其余同任务条目（可用，条件较多）

| 文献 / 出处 | 设置 | 可用数值 | 备注 |
| --- | --- | --- | --- |
| WPD-attention-LSTM Wiley, DOI 10.1155/vib/8680245 | SEU 齿轮 5 类 · 5240 样本 · 7:3 · 重复 5 次 | ELM 83.9 / 1DCNN 83.6 / SVM 89.7 | 1DCNN 后带引用号 [33] ， 可能转引 ，须核 |
| MSCNN-ViT IEEE AiDAS 2025 | SEU 齿轮 5 类 · 每类 300 · 本文 99.83% | SVM ≈78.51 / 2D-CNN ≈89.71 / ResNet-50 ≈92.30 / DenseNet-121 ≈93.13 / ViT-Base ≈94.22 | 由"提升量"反推，非原表数值 ，须核 |
| MSSLWF Springer LNEE 2025 | SEU 齿轮 5 类 · 两工况 · 8:2 · 8000/2000 · 5 次 | X 向 99.92 / Y 向 99.11 / 非自学习融合 99.75 | 本文 99.97 ，差距极小 |
| GSWOA-KELM Lubricants 2024, 12(1):10 | SEU 齿轮 20-0 · 5 类 · 每类 100 （ 2048 点） · 7:3 | 时域特征 86.67 / 频域特征 85.33 （融合后 100 ） | 可作 " 特征工程对比 " |
| RTSMRDE + SSA-SVM Machines 2023, 11(6):646 | SEU 齿轮 20-0 · Y 向 · 5 类 × 50 子样本 · 10 训 / 40 测 | SSA-SVM 100% | 样本量仅 250 ，只能作参照 |
| Swin Transformer + MTF Eng. Res. Express 2025, 7:015225 | DDS 齿轮 | 本文 99.69% ；优于 CWT / GAF / CNN / ViT | 对照数值待补 |
| DSMC-ECA Sensors 2026, 26(4):1196 | SEU 齿轮 · 强噪声 · 0.204 M 参数 | −6 dB 下 86.84% | 轻量化方向，可呼应效率节 |
| He et al. 迁移学习 NDT&E 2025 | SEU 变工况齿轮箱 | 平均 96.12% | 变工况，难度不同级 |

## 三、轴承 · 单工况 5 分类

### 3.1  Dual-Channel 并行多模态融合 ★★

Li W., Cai H., Yang X., Xue Y., Ye J., Hu X. Dual-Channel Parallel Multimodal Feature Fusion for Bearing Fault Diagnosis. Machines, 2025, 13(10): 950. DOI: 10.3390/machines13100950

| 方法 | −6 dB | −4 dB | 0 dB | 4 dB | 6 dB | 无噪声 |
| --- | --- | --- | --- | --- | --- | --- |
| MLP （ BP ） | 59.67 | 61.67 | 70.00 | 76.00 | 77.33 | — |
| CNN | 72.67 | 73.27 | 78.13 | 81.47 | 82.73 | — |
| LSTM | 74.33 | 75.00 | 78.33 | 82.47 | 83.93 | — |
| GRU | 74.93 | 76.93 | 82.07 | 83.27 | 84.47 | — |
| 本文方法 | 85.87 | 86.73 | 91.67 | 93.27 | 94.67 | 97.33 |

设置：SEU 轴承、30 Hz-2 V（1800 rpm, 7.32 N·m）单工况、5 类（ball / compound / health / inner / outer）；采样 5120 Hz；样本长 1024、重叠 512；每类 300（共 1500）；6:2:2 划分；加性高斯白噪声；每个点 5 次实验取均值。无交叉验证，但报了标准差。这是轴承侧最干净的一条。

### 3.2  ADSRN（小样本）

Jiang Y., Lu M., Dong Z., et al. Adaptive Deeping Siamese Residual Network: A Novel Model for Few-Shot Bearing Fault Diagnosis. Machines, 2025, 13(3): 193. DOI: 10.3390/machines13030193

| 方法 | 10 样本 / 类 | 30 样本 / 类 |
| --- | --- | --- |
| ResNet | 75.4 | 75.50 ± 2.32 |
| WDCNN | 46.42 | 81.06 ± 4.55 |
| CNN-LSTM | 83.55 | 96.35 ± 2.41 |
| ADSRN （本文方法） | 96.61 | 98.85 ± 0.23 |

只能用于小样本场景对比，与本文全样本设置不同级。

### 3.3  其余轴承条目

| 文献 / 出处 | 任务 | 可用数值 |
| --- | --- | --- |
| CAM 交叉注意力 Electronics 2025, 14(5):886 | SEU 轴承 10 类 （ 20_0 与 30_2 混合） | CNN 90.76 / MCNN-LSTM 98.36 / BAN 99.49 / CAM 99.98 |
| MGE-ResNet Processes 2025, 13(4):1239 | SEU 轴承 10 类 | SVM 81.46 ± 4.28 / TICNN 88.50 ± 8.14 / MGE-ResNet 99.44 |
| CNN-Informer-DA Meas. Sci. Technol. 2025, 37(2) | SEU 轴承 | 98.44% |
| MSWCTD Sensors 2025, 25(10):3141 | SEU 轴承 3 类 跨机器迁移 | 100% / 97.67% |
| 模态融合深度聚类 电子与信息学报 2025, 47(1):244 | SEU 齿轮 / 轴承（无监督） | 齿轮 99.16% / 轴承 98.63% |

## 四、低分但必须加条件的条目

| 文献 | 可用低分 | 必须写明 |
| --- | --- | --- |
| SVMD 熵 Algorithms 2023, 16(6):304 | 齿轮 89.41~96.02% ；轴承 30-2 94.12~97.25% | 原文明确 " 无训练集，全部数据用于测试 "—— 样本内拟合值，非泛化准确率 |
| IAMCNN 华东理工大学学报 2024, 50(6):920-928 | BP 82.03 / CNN 85.46 / LSTM 90.29 / ResNet 95.35 | 只能取表 1 （表 2 是行星齿轮箱）；无 CV |
| EBRB-EU Sci. Rep. 2026 | IBRB 82.0 / BRF 81.7 / ABRB 82.6 / CSL 83.2 / CWSVM 84.0 / HBRB 84.1 / BPNN 86.2 （本文 92.8 ） | 跨工况（20→30）+ 类别不平衡 ，难度与单工况 5 类不同级 |

EBRB-EU 是全库定位最接近"80 多分"的一篇——但它同时换了工况和采样不平衡设置，只能作"更困难场景下仍有方法落在 82~86%"的旁证，不能进主表。

## 五、同库 · 任务不同（只能作量级参照）

Kibrete 1D CNN-LSTM（Wiley J. Engineering 2025, DOI 10.1155/je/1670810，变工况组合集 10 类，CNN 94.12 / LSTM 94.90 / 1D CNN 97.67）；ACM 多模态融合 2025（ACM CIISAI，SEU 轴承 CNN 89.50 / Transformer 63.56；SEU 齿轮 CNN 97.27 / Transformer 71.08）；ADAMFFN（Machines 2026，齿轮跨工况 94.54%）；SAGAN-IResNet（Sci. Rep. 2025，8:1:1 划分 + CWT 时频图）；IIETA MSRC+CBAM（60%/30% 划分）。

## 六、必须撤下 / 慎用

FE-MCFormer（arXiv:2505.06285）：v2 版含 SEU 齿轮箱案例（5 类两工况、5000 样本、4000 / 1000），但 v3 当前版已整段删除（改为 PU 轴承 + 离心压缩机）。它那张强噪声表（MSCNN-LSTM 28.16 / WDCNN 66.16 / ResNet50 78.54）确实存在过，但已被作者删除。→ 建议弃用；若一定要用，必须写明"引自 v2"。

各类 CSDN / 公众号博客里的东南大学数据集结果（如 WSET-CNN-BKA-LSSVM、GADF-CNN-SSA-XGBoost）：非正式发表文献，不能引用。

## 七、出处登记

1. Ma R., et al. DDGF-Net: A Novel Dual-Domain Generative-Discriminative Fusion Network for Gearbox Fault Diagnosis Under Strong Noise Conditions. Machines, 2026, 14(9): 1013. DOI: 10.3390/machines14091013

2. Zhu C., Chen Z., Zhao R., Wang J., Yan R. Decoupled Feature-Temporal CNN: Explaining Deep Learning-Based Machine Health Monitoring. IEEE Trans. Instrum. Meas., 2021, 70: 1–13. DOI: 10.1109/TIM.2021.3084310

3. Fault Diagnosis of Gearbox Based on CNN and LSTM with Attention Mechanism. Highlights in Science, Engineering and Technology, 2024, 93: 291–292.

4. Wang Z., Tang Y., Dong Y., et al. Optimization of Gearbox Fault Detection Method Based on Deep Residual Neural Network Algorithm. Sensors, 2023, 23(17): 7573. DOI: 10.3390/s23177573

5. Fault Classification Method for Rotating Machinery Based on Hybrid Model of CWT and CNN-DOA-LSSVM. Information, 2026, 17(6): 580. DOI: 10.3390/info17060580

6. Gearbox Fault Diagnosis Based on Compressed Sensing and Multi-Scale Residual Network with Lightweight Attention Mechanism. Mathematics, 2025, 13(9): 1393. DOI: 10.3390/math13091393

7. Hierarchical Convolution-Transformer Framework for Gear Fault Diagnosis Under Severe Noise. IEEE Xplore, 2025, 文献号 11037744.

8. Li W., Cai H., Yang X., Xue Y., Ye J., Hu X. Dual-Channel Parallel Multimodal Feature Fusion for Bearing Fault Diagnosis. Machines, 2025, 13(10): 950. DOI: 10.3390/machines13100950

9. Jiang Y., Lu M., Dong Z., et al. Adaptive Deeping Siamese Residual Network: A Novel Model for Few-Shot Bearing Fault Diagnosis. Machines, 2025, 13(3): 193. DOI: 10.3390/machines13030193

10. Wu N., Li Y., Li X., Yuan K., Jiang W. An Investigation on Attention Mechanism–Based Long Short-Term Memory for Gearbox Fault Diagnosis. Wiley. DOI: 10.1155/vib/8680245

11. Duan L., et al. Gearbox Fault Diagnosis Method Based On MSCNN-ViT. 2025 6th Int. Conf. on Artificial Intelligence and Data Sciences (AiDAS). DOI: 10.1109/AiDAS67696.2025.11213541

12. Gearbox Fault Diagnosis Method Based on Model Transfer and Multi-sensor Self-learning Weighted Fusion. Springer LNEE, 2025. DOI: 10.1007/978-981-96-5527-4_28

13. Time-Frequency Fusion Features-Based GSWOA-KELM Model for Gear Fault Diagnosis. Lubricants, 2024, 12(1): 10. DOI: 10.3390/lubricants12010010

14. Gearbox Fault Diagnosis Based on Refined Time-Shift Multiscale Reverse Dispersion Entropy and Optimised Support Vector Machine. Machines, 2023, 11(6): 646. DOI: 10.3390/machines11060646

15. Liu J., Hao R., Deng F., Fan H., Lou H. A gearbox fault diagnosis method based on Swin Transformer and Markov transform fields. Eng. Res. Express, 2025, 7: 015225. DOI: 10.1088/2631-8695/ada71f

16. Lightweight Gearbox Fault Diagnosis Under High Noise Based on Improved Multi-Scale Depthwise Separable Convolution and Efficient Channel Attention. Sensors, 2026, 26(4): 1196. DOI: 10.3390/s26041196

17. He X., Zhao F., et al. A deep-transfer-learning fault diagnosis method for gearboxes based on discriminative feature extraction and improved domain adversarial neural networks. Nondestructive Testing and Evaluation, 2025. DOI: 10.1080/10589759.2025.2495802

18. A Novel Framework Based on Complementary Views for Fault Diagnosis with Cross-Attention Mechanisms. Electronics, 2025, 14(5): 886. DOI: 10.3390/electronics14050886

19. Bearing Fault Diagnosis Based on Multiscale Lightweight Convolutional Neural Network. Processes, 2025, 13(4): 1239. DOI: 10.3390/pr13041239

20. Wang Y., Hui M., Sun J., Ling D., Wang Y. Rolling bearing health status diagnosis method based on bidirectional feature extraction by fusing CNN and Informer. Meas. Sci. Technol., 2025, 37(2). DOI: 10.1088/1361-6501/ae2d7e

21. A Novel Multistep Wavelet Convolutional Transfer Diagnostic Framework for Cross-Machine Bearing Fault Diagnosis. Sensors, 2025, 25(10): 3141. DOI: 10.3390/s25103141

22. 伍章俊, 许仁礼, 方刚, 邵海东. 一种面向旋转机械多传感器故障诊断的模态融合深度聚类方法. 电子与信息学报, 2025, 47(1): 244. DOI: 10.11999/JEIT240648

23. Zhang L., Zhang Y., Li G. Fault-Diagnosis Method for Rotating Machinery Based on SVMD Entropy and Machine Learning. Algorithms, 2023, 16(6): 304. DOI: 10.3390/a16060304

24. 邵浙梁, 戚知宽, 周邵萍. 基于改进注意力机制的 CNN 的齿轮箱故障诊断. 华东理工大学学报（自然科学版）, 2024, 50(6): 920-928. DOI: 10.14135/j.cnki.1006-3080.20231207002

25. A bearing fault diagnosis method for complex system based on improved extended belief rule base. Sci. Rep., 2026. DOI: 10.1038/s41598-026-44629-8

26. Kibrete F., Woldemichael D.E., Gebremedhen H.S. Fault Diagnosis of Rotating Machines Based on Combination of One-Dimensional Convolutional Neural Network and Long Short-Term Memory in Variable Working Conditions. J. Engineering, 2025. DOI: 10.1155/je/1670810

27. Research on Equipment Fault Diagnosis Method Based on Multimodal Deep Fusion. ACM CIISAI 2025. DOI: 10.1145/3773365.3773555